# Assignment 1

# Christopher Budd

# 218919068

## Question 1

## Question 2

### Pictures

### Code

Build_tower.py

import sys

import os

import time

import threading

from kortex_api.autogen.client_stubs.BaseClientRpc import BaseClient

from kortex_api.autogen.client_stubs.BaseCyclicClientRpc import BaseCyclicClient

from kortex_api.autogen.messages import Base_pb2, BaseCyclic_pb2, Common_pb2

# Maximum allowed waiting time during actions (in seconds)

TIMEOUT_DURATION = 20

# Create closure to set an event after an END or an ABORT

def check_for_end_or_abort(e):

"""Return a closure checking for END or ABORT notifications

Arguments:

e -- event to signal when the action is completed

(will be set when an END or ABORT occurs)

"""

def check(notification, e = e):

print("EVENT : " + \

Base_pb2.ActionEvent.Name(notification.action_event))

if notification.action_event == Base_pb2.ACTION_END \

or notification.action_event == Base_pb2.ACTION_ABORT:

e.set()

return check

def set_gripper(base, position):

gripper_command = Base_pb2.GripperCommand()

finger = gripper_command.gripper.finger.add()

# Close the gripper with position increments

print("Performing gripper test in position...")

gripper_command.mode = Base_pb2.GRIPPER_POSITION

finger.value = position

print(f"Going to position {position}")

base.SendGripperCommand(gripper_command)

def get_gripper(base):

gripper_request = Base_pb2.GripperRequest()

gripper_request.mode = Base_pb2.GRIPPER_POSITION

gripper_measure = base.GetMeasuredGripperMovement(gripper_request)

if len (gripper_measure.finger):

print(f"Current position is : {gripper_measure.finger[0].value}")

return gripper_measure.finger[0].value

return None

def example_move_to_home_position(base):

# Make sure the arm is in Single Level Servoing mode

base_servo_mode = Base_pb2.ServoingModeInformation()

base_servo_mode.servoing_mode = Base_pb2.SINGLE_LEVEL_SERVOING

base.SetServoingMode(base_servo_mode)

# Move arm to ready position

print("Moving the arm to a safe position")

action_type = Base_pb2.RequestedActionType()

action_type.action_type = Base_pb2.REACH_JOINT_ANGLES

action_list = base.ReadAllActions(action_type)

action_handle = None

for action in action_list.action_list:

if action.name == "Home":

action_handle = action.handle

if action_handle == None:

print("Can't reach safe position. Exiting")

return False

e = threading.Event()

notification_handle = base.OnNotificationActionTopic(

check_for_end_or_abort(e),

Base_pb2.NotificationOptions()

)

base.ExecuteActionFromReference(action_handle)

finished = e.wait(TIMEOUT_DURATION)

base.Unsubscribe(notification_handle)

if finished:

print("Safe position reached")

else:

print("Timeout on action notification wait")

return finished

def example_angular_action_movement(base, angles=[0.0, 0.0, 0.0, 0.0, 0.0, 0.0]):

print("Starting angular action movement ...")

action = Base_pb2.Action()

action.name = "Example angular action movement"

action.application_data = ""

actuator_count = base.GetActuatorCount()

# Place arm straight up

print(actuator_count.count)

if actuator_count.count != len(angles):

print(f"bad lengths {actuator_count.count} {len(angles)}")

for joint_id in range(actuator_count.count):

joint_angle = action.reach_joint_angles.joint_angles.joint_angles.add()

joint_angle.joint_identifier = joint_id

joint_angle.value = angles[joint_id]

e = threading.Event()

notification_handle = base.OnNotificationActionTopic(

check_for_end_or_abort(e),

Base_pb2.NotificationOptions()

)

print("Executing action")

base.ExecuteAction(action)

print("Waiting for movement to finish ...")

finished = e.wait(TIMEOUT_DURATION)

base.Unsubscribe(notification_handle)

if finished:

print("Angular movement completed")

else:

print("Timeout on action notification wait")

return finished

def example_cartesian_action_movement(base, base_cyclic):

print("Starting Cartesian action movement ...")

action = Base_pb2.Action()

action.name = "Example Cartesian action movement"

action.application_data = ""

feedback = base_cyclic.RefreshFeedback()

cartesian_pose = action.reach_pose.target_pose

cartesian_pose.x = feedback.base.tool_pose_x          # (meters)

cartesian_pose.y = feedback.base.tool_pose_y - 0.1    # (meters)

cartesian_pose.z = feedback.base.tool_pose_z - 0.2    # (meters)

cartesian_pose.theta_x = feedback.base.tool_pose_theta_x # (degrees)

cartesian_pose.theta_y = feedback.base.tool_pose_theta_y # (degrees)

cartesian_pose.theta_z = feedback.base.tool_pose_theta_z # (degrees)

e = threading.Event()

notification_handle = base.OnNotificationActionTopic(

check_for_end_or_abort(e),

Base_pb2.NotificationOptions()

)

print("Executing action")

base.ExecuteAction(action)

print("Waiting for movement to finish ...")

finished = e.wait(TIMEOUT_DURATION)

base.Unsubscribe(notification_handle)

if finished:

print("Cartesian movement completed")

else:

print("Timeout on action notification wait")

return finished

def main():

sys.path.insert(0, os.path.join(os.path.dirname(__file__), ".."))

import utilities

# Parse arguments

args = utilities.parseConnectionArguments()

# Create connection to the device and get the router

with utilities.DeviceConnection.createTcpConnection(args) as router:

# Create required services

base = BaseClient(router)

base_cyclic = BaseCyclicClient(router)

success = True

set_gripper(base, 0.0)

time.sleep(2)

success &= example_move_to_home_position(base)

#First Block

success &= example_angular_action_movement(base, [48,-58,78,0,0,0])

set_gripper(base, 1.0)

time.sleep(2)

success &= example_angular_action_movement(base, [0,0,78,0,0,0])

# Stack Spot

success &= example_angular_action_movement(base, [0,-58,78,0,0,0])

set_gripper(base, 0.0)

time.sleep(2)

# No hit

success &= example_angular_action_movement(base, [0,0,60,0,0,0])

# 2 Block

success &= example_angular_action_movement(base, [-4,-37,126,0,0,0])

set_gripper(base, 1.0)

time.sleep(2)

success &= example_angular_action_movement(base, [0,0,126,0,0,0])

# Stack Spot

success &= example_angular_action_movement(base, [0,-52,83,0,0,0])

set_gripper(base, 0.0)

time.sleep(2)

# Reset For next

success &= example_angular_action_movement(base, [0,-40,80,0,0,0])

# 3 Block

success &= example_angular_action_movement(base, [-9,-47,98,0,0,30])

set_gripper(base, 1.0)

time.sleep(2)

# Reset and setup

success &= example_angular_action_movement(base, [0,-40,80,0,0,0])

# Last Drop

success &= example_angular_action_movement(base, [0,-47,84,0,0,0])

set_gripper(base, 0.0)

time.sleep(2)

return 0 if success else 1

if __name__ == "__main__":

exit(main())

### How successful were you in this task?

I would say 70% successful as although it does work, it could be better. The tower itself was stable enough but if I had adjusted the arm more, it would have been better.

## Question 3

### Pictures

### Code

import math

import numpy as np

import rclpy

from rclpy.node import Node

from rclpy.parameter import Parameter

from rcl_interfaces.msg import SetParametersResult

from nav_msgs.msg import Odometry

from geometry_msgs.msg import Twist, Pose, Point, Quaternion

from nav_msgs.msg import Odometry

def euler_from_quaternion(quaternion):

"""

Converts quaternion (w in last place) to euler roll, pitch, yaw

quaternion = [x, y, z, w]

"""

x = quaternion.x

y = quaternion.y

z = quaternion.z

w = quaternion.w

sinr_cosp = 2 * (w * x + y * z)

cosr_cosp = 1 - 2 * (x * x + y * y)

roll = np.arctan2(sinr_cosp, cosr_cosp)

sinp = 2 * (w * y - z * x)

pitch = np.arcsin(sinp)

siny_cosp = 2 * (w * z + x * y)

cosy_cosp = 1 - 2 * (y * y + z * z)

yaw = np.arctan2(siny_cosp, cosy_cosp)

return roll, pitch, yaw

class MoveToGoal(Node):

def __init__(self):

super().__init__('move_robot_to_goal')

self.get_logger().info(f'{self.get_name()} created')

self.declare_parameter('goal_x', 0.0)

self._goal_x = self.get_parameter('goal_x').get_parameter_value().double_value

self.declare_parameter('goal_y', 0.0)

self._goal_y = self.get_parameter('goal_y').get_parameter_value().double_value

self.declare_parameter('goal_t', 0.0)

self._goal_t = self.get_parameter('goal_t').get_parameter_value().double_value

self.declare_parameter('max_vel', 0.4)

self._max_vel = self.get_parameter('max_vel').get_parameter_value().double_value

self.declare_parameter('cmd_gain', 5.0)

self._cmd_gain = self.get_parameter('cmd_gain').get_parameter_value().double_value

self.add_on_set_parameters_callback(self.parameter_callback)

self.get_logger().info(f"initial goal {self._goal_x} {self._goal_y} {self._goal_t}")

self.get_logger().info(f"maximum velocity {self._max_vel}")

self._subscriber = self.create_subscription(Odometry, "/odom", self._listener_callback, 1)

self._publisher = self.create_publisher(Twist, "/cmd_vel", 1)

def _listener_callback(self, msg, vel_gain=5.0, max_vel=0.2, max_pos_err=0.05):

pose = msg.pose.pose

max_vel = self._max_vel

vel_gain = self._cmd_gain

cur_x = pose.position.x

cur_y = pose.position.y

o = pose.orientation

roll, pitchc, yaw = euler_from_quaternion(o)

cur_t = yaw

x_diff = self._goal_x - cur_x

y_diff = self._goal_y - cur_y

dist = math.sqrt(x_diff * x_diff + y_diff * y_diff)

twist = Twist()

if dist > max_pos_err:

x = max(min(x_diff * vel_gain, max_vel), -max_vel)

y = max(min(y_diff * vel_gain, max_vel), -max_vel)

twist.linear.x = x * math.cos(cur_t) + y * math.sin(cur_t)

twist.linear.y = -x * math.sin(cur_t) + y * math.cos(cur_t)

angle_diff = math.atan2(math.sin(self._goal_t - cur_t), math.cos(self._goal_t - cur_t))

if abs(angle_diff) > max_pos_err:

self.get_logger().info(f"Twist {angle_diff}")

twist.angular.z = max(min(angle_diff * vel_gain*4, max_vel*5), -max_vel*5)

self.get_logger().info(f"Twist ang {twist.angular.z}")

self.get_logger().info(f"at ({cur_x},{cur_y},{cur_t}) goal ({self._goal_x},{self._goal_y},{self._goal_t})")

self._publisher.publish(twist)

def parameter_callback(self, params):

self.get_logger().info(f'move_robot_to_goal parameter callback {params}')

for param in params:

self.get_logger().info(f'move_robot_to_goal processing {param.name}')

if param.name == 'goal_x' and param.type_ == Parameter.Type.DOUBLE:

self._goal_x = param.value

elif param.name == 'goal_y' and param.type_ == Parameter.Type.DOUBLE:

self._goal_y = param.value

elif param.name == 'goal_t' and param.type_ == Parameter.Type.DOUBLE:

self._goal_t = param.value

else:

self.get_logger().warn(f'{self.get_name()} Invalid parameter {param.name}')

return SetParametersResult(successful=False)

self.get_logger().warn(f"Changing goal {self._goal_x} {self._goal_y} {self._goal_t}")

return SetParametersResult(successful=True)

def main(args=None):

rclpy.init(args=args)

node = MoveToGoal()

try:

rclpy.spin(node)

except KeyboardInterrupt:

pass

rclpy.shutdown()

if __name__ == '__main__':

main()

## Question 4

### Layout

If not at goal(if dist > max_pos_err)

Drive towards goal

For object in object list

If an object in the way of the robot within a range

Follow the obstacle boundary moving to the left or right.
or based on the lab 1 code

For o in map

Check if in range and in the way

InWay = true

If inway

Follow the obstacle boundary

Else

Drive towards goal

### Explanation

Using _listener_callback, if the robot has not reached its goal, move towards the goal until an object is in its way. It checks the list when it moves to simulate it discovering the object by sensors or hitting it. It then follows the object boundary until it can move towards the goal.

## Question 5

### Pictures

### Code

def _listener_callback(self, msg, vel_gain=5.0, max_vel=0.2, max_pos_err=0.05):

pose = msg.pose.pose

max_vel = self._max_vel

vel_gain = self._cmd_gain

cur_x = pose.position.x

cur_y = pose.position.y

o = pose.orientation

roll, pitchc, yaw = euler_from_quaternion(o)

cur_t = yaw

x_diff = self._goal_x - cur_x

y_diff = self._goal_y - cur_y

dist = math.sqrt(x_diff * x_diff + y_diff * y_diff)

twist = Twist()

obstacle_detected = False

for o in self._map.keys():

obj_x, obj_y, obj_r = self._map[o]['x'], self._map[o]['y'], self._map[o]['r']

obj_dist = math.sqrt((cur_x - obj_x)**2 + (cur_y - obj_y)**2)

self.get_logger().info(f"Obstacle States {o} {obj_dist}")

if obj_dist <= obj_r + 0.3:  # 0.5 is a safety margin

obstacle_detected = True

break

if obstacle_detected:

angle_to_obj = math.atan2(obj_y - cur_y, obj_x - cur_x)

tangent_angle = angle_to_obj + math.pi/2

x_tangent = math.cos(tangent_angle)

y_tangent = math.sin(tangent_angle)

x = max(min(x_tangent * vel_gain, max_vel), -max_vel)

y = max(min(y_tangent * vel_gain, max_vel), -max_vel)

twist.linear.x = x * math.cos(cur_t) + y * math.sin(cur_t)

twist.linear.y = -x * math.sin(cur_t) + y * math.cos(cur_t)

self.get_logger().info(f"Obstacle detected")

elif dist > max_pos_err:

x = max(min(x_diff * vel_gain, max_vel), -max_vel)

y = max(min(y_diff * vel_gain, max_vel), -max_vel)

twist.linear.x = x * math.cos(cur_t) + y * math.sin(cur_t)

twist.linear.y = -x * math.sin(cur_t) + y * math.cos(cur_t)

angle_diff = math.atan2(math.sin(self._goal_t - cur_t), math.cos(self._goal_t - cur_t))

if abs(angle_diff) > max_pos_err:

self.get_logger().info(f"Twist {angle_diff}")

twist.angular.z = max(min(angle_diff * vel_gain*4, max_vel*5), -max_vel*5)

self.get_logger().info(f"Twist ang {twist.angular.z}")

self.get_logger().info(f"at ({cur_x},{cur_y},{cur_t}) goal ({self._goal_x},{self._goal_y},{self._goal_t})")

self._publisher.publish(twist)