# Assignment 1

# Christopher Budd

# 218919068

## Question 1

## Question 2

### Pictures

### Code

Build_tower.py

import sys

import os

import time

import threading

from kortex_api.autogen.client_stubs.BaseClientRpc import BaseClient

from kortex_api.autogen.client_stubs.BaseCyclicClientRpc import BaseCyclicClient

from kortex_api.autogen.messages import Base_pb2, BaseCyclic_pb2, Common_pb2

# Maximum allowed waiting time during actions (in seconds)

TIMEOUT_DURATION = 20

# Create closure to set an event after an END or an ABORT

def check_for_end_or_abort(e):

"""Return a closure checking for END or ABORT notifications

Arguments:

e -- event to signal when the action is completed

(will be set when an END or ABORT occurs)

"""

def check(notification, e = e):

print("EVENT : " + \

Base_pb2.ActionEvent.Name(notification.action_event))

if notification.action_event == Base_pb2.ACTION_END \

or notification.action_event == Base_pb2.ACTION_ABORT:

e.set()

return check

def set_gripper(base, position):

gripper_command = Base_pb2.GripperCommand()

finger = gripper_command.gripper.finger.add()

# Close the gripper with position increments

print("Performing gripper test in position...")

gripper_command.mode = Base_pb2.GRIPPER_POSITION

finger.value = position

print(f"Going to position {position}")

base.SendGripperCommand(gripper_command)

def get_gripper(base):

gripper_request = Base_pb2.GripperRequest()

gripper_request.mode = Base_pb2.GRIPPER_POSITION

gripper_measure = base.GetMeasuredGripperMovement(gripper_request)

if len (gripper_measure.finger):

print(f"Current position is : {gripper_measure.finger[0].value}")

return gripper_measure.finger[0].value

return None

def example_move_to_home_position(base):

# Make sure the arm is in Single Level Servoing mode

base_servo_mode = Base_pb2.ServoingModeInformation()

base_servo_mode.servoing_mode = Base_pb2.SINGLE_LEVEL_SERVOING

base.SetServoingMode(base_servo_mode)

# Move arm to ready position

print("Moving the arm to a safe position")

action_type = Base_pb2.RequestedActionType()

action_type.action_type = Base_pb2.REACH_JOINT_ANGLES

action_list = base.ReadAllActions(action_type)

action_handle = None

for action in action_list.action_list:

if action.name == "Home":

action_handle = action.handle

if action_handle == None:

print("Can't reach safe position. Exiting")

return False

e = threading.Event()

notification_handle = base.OnNotificationActionTopic(

check_for_end_or_abort(e),

Base_pb2.NotificationOptions()

)

base.ExecuteActionFromReference(action_handle)

finished = e.wait(TIMEOUT_DURATION)

base.Unsubscribe(notification_handle)

if finished:

print("Safe position reached")

else:

print("Timeout on action notification wait")

return finished

def example_angular_action_movement(base, angles=[0.0, 0.0, 0.0, 0.0, 0.0, 0.0]):

print("Starting angular action movement ...")

action = Base_pb2.Action()

action.name = "Example angular action movement"

action.application_data = ""

actuator_count = base.GetActuatorCount()

# Place arm straight up

print(actuator_count.count)

if actuator_count.count != len(angles):

print(f"bad lengths {actuator_count.count} {len(angles)}")

for joint_id in range(actuator_count.count):

joint_angle = action.reach_joint_angles.joint_angles.joint_angles.add()

joint_angle.joint_identifier = joint_id

joint_angle.value = angles[joint_id]

e = threading.Event()

notification_handle = base.OnNotificationActionTopic(

check_for_end_or_abort(e),

Base_pb2.NotificationOptions()

)

print("Executing action")

base.ExecuteAction(action)

print("Waiting for movement to finish ...")

finished = e.wait(TIMEOUT_DURATION)

base.Unsubscribe(notification_handle)

if finished:

print("Angular movement completed")

else:

print("Timeout on action notification wait")

return finished

def example_cartesian_action_movement(base, base_cyclic):

print("Starting Cartesian action movement ...")

action = Base_pb2.Action()

action.name = "Example Cartesian action movement"

action.application_data = ""

feedback = base_cyclic.RefreshFeedback()

cartesian_pose = action.reach_pose.target_pose

cartesian_pose.x = feedback.base.tool_pose_x          # (meters)

cartesian_pose.y = feedback.base.tool_pose_y - 0.1    # (meters)

cartesian_pose.z = feedback.base.tool_pose_z - 0.2    # (meters)

cartesian_pose.theta_x = feedback.base.tool_pose_theta_x # (degrees)

cartesian_pose.theta_y = feedback.base.tool_pose_theta_y # (degrees)

cartesian_pose.theta_z = feedback.base.tool_pose_theta_z # (degrees)

e = threading.Event()

notification_handle = base.OnNotificationActionTopic(

check_for_end_or_abort(e),

Base_pb2.NotificationOptions()

)

print("Executing action")

base.ExecuteAction(action)

print("Waiting for movement to finish ...")

finished = e.wait(TIMEOUT_DURATION)

base.Unsubscribe(notification_handle)

if finished:

print("Cartesian movement completed")

else:

print("Timeout on action notification wait")

return finished

def main():

sys.path.insert(0, os.path.join(os.path.dirname(__file__), ".."))

import utilities

# Parse arguments

args = utilities.parseConnectionArguments()

# Create connection to the device and get the router

with utilities.DeviceConnection.createTcpConnection(args) as router:

# Create required services

base = BaseClient(router)

base_cyclic = BaseCyclicClient(router)

success = True

set_gripper(base, 0.0)

time.sleep(2)

success &= example_move_to_home_position(base)

#First Block

success &= example_angular_action_movement(base, [48,-58,78,0,0,0])

set_gripper(base, 1.0)

time.sleep(2)

success &= example_angular_action_movement(base, [0,0,78,0,0,0])

# Stack Spot

success &= example_angular_action_movement(base, [0,-58,78,0,0,0])

set_gripper(base, 0.0)

time.sleep(2)

# No hit

success &= example_angular_action_movement(base, [0,0,60,0,0,0])

# 2 Block

success &= example_angular_action_movement(base, [-4,-37,126,0,0,0])

set_gripper(base, 1.0)

time.sleep(2)

success &= example_angular_action_movement(base, [0,0,126,0,0,0])

# Stack Spot

success &= example_angular_action_movement(base, [0,-52,83,0,0,0])

set_gripper(base, 0.0)

time.sleep(2)

# Reset For next

success &= example_angular_action_movement(base, [0,-40,80,0,0,0])

# 3 Block

success &= example_angular_action_movement(base, [-9,-47,98,0,0,30])

set_gripper(base, 1.0)

time.sleep(2)

# Reset and setup

success &= example_angular_action_movement(base, [0,-40,80,0,0,0])

# Last Drop

success &= example_angular_action_movement(base, [0,-47,84,0,0,0])

set_gripper(base, 0.0)

time.sleep(2)

return 0 if success else 1

if __name__ == "__main__":

exit(main())

### How successful were you in this task?

I would say 70% successful as although it does work, it could be better. The tower itself was stable enough but if I had adjusted the arm more, it would have been better.

## Question 3

### Pictures

### Code

import math

import numpy as np

import rclpy

from rclpy.node import Node

from rclpy.parameter import Parameter

from rcl_interfaces.msg import SetParametersResult

from nav_msgs.msg import Odometry

from geometry_msgs.msg import Twist, Pose, Point, Quaternion

from nav_msgs.msg import Odometry

def euler_from_quaternion(quaternion):

"""

Converts quaternion (w in last place) to euler roll, pitch, yaw

quaternion = [x, y, z, w]

"""

x = quaternion.x

y = quaternion.y

z = quaternion.z

w = quaternion.w

sinr_cosp = 2 * (w * x + y * z)

cosr_cosp = 1 - 2 * (x * x + y * y)

roll = np.arctan2(sinr_cosp, cosr_cosp)

sinp = 2 * (w * y - z * x)

pitch = np.arcsin(sinp)

siny_cosp = 2 * (w * z + x * y)

cosy_cosp = 1 - 2 * (y * y + z * z)

yaw = np.arctan2(siny_cosp, cosy_cosp)

return roll, pitch, yaw

class MoveToGoal(Node):

def __init__(self):

super().__init__('move_robot_to_goal')

self.get_logger().info(f'{self.get_name()} created')

self.declare_parameter('goal_x', 0.0)

self._goal_x = self.get_parameter('goal_x').get_parameter_value().double_value

self.declare_parameter('goal_y', 0.0)

self._goal_y = self.get_parameter('goal_y').get_parameter_value().double_value

self.declare_parameter('goal_t', 0.0)

self._goal_t = self.get_parameter('goal_t').get_parameter_value().double_value

self.declare_parameter('max_vel', 0.4)

self._max_vel = self.get_parameter('max_vel').get_parameter_value().double_value

self.declare_parameter('cmd_gain', 5.0)

self._cmd_gain = self.get_parameter('cmd_gain').get_parameter_value().double_value

self.add_on_set_parameters_callback(self.parameter_callback)

self.get_logger().info(f"initial goal {self._goal_x} {self._goal_y} {self._goal_t}")

self.get_logger().info(f"maximum velocity {self._max_vel}")

self._subscriber = self.create_subscription(Odometry, "/odom", self._listener_callback, 1)

self._publisher = self.create_publisher(Twist, "/cmd_vel", 1)

def _listener_callback(self, msg, vel_gain=5.0, max_vel=0.2, max_pos_err=0.05):

pose = msg.pose.pose

max_vel = self._max_vel

vel_gain = self._cmd_gain

cur_x = pose.position.x

cur_y = pose.position.y

o = pose.orientation

roll, pitchc, yaw = euler_from_quaternion(o)

cur_t = yaw

x_diff = self._goal_x - cur_x

y_diff = self._goal_y - cur_y

dist = math.sqrt(x_diff * x_diff + y_diff * y_diff)

twist = Twist()

if dist > max_pos_err:

x = max(min(x_diff * vel_gain, max_vel), -max_vel)

y = max(min(y_diff * vel_gain, max_vel), -max_vel)

twist.linear.x = x * math.cos(cur_t) + y * math.sin(cur_t)

twist.linear.y = -x * math.sin(cur_t) + y * math.cos(cur_t)

angle_diff = math.atan2(math.sin(self._goal_t - cur_t), math.cos(self._goal_t - cur_t))

if abs(angle_diff) > max_pos_err:

self.get_logger().info(f"Twist {angle_diff}")

twist.angular.z = max(min(angle_diff * vel_gain*4, max_vel*5), -max_vel*5)

self.get_logger().info(f"Twist ang {twist.angular.z}")

self.get_logger().info(f"at ({cur_x},{cur_y},{cur_t}) goal ({self._goal_x},{self._goal_y},{self._goal_t})")

self._publisher.publish(twist)

def parameter_callback(self, params):

self.get_logger().info(f'move_robot_to_goal parameter callback {params}')

for param in params:

self.get_logger().info(f'move_robot_to_goal processing {param.name}')

if param.name == 'goal_x' and param.type_ == Parameter.Type.DOUBLE:

self._goal_x = param.value

elif param.name == 'goal_y' and param.type_ == Parameter.Type.DOUBLE:

self._goal_y = param.value

elif param.name == 'goal_t' and param.type_ == Parameter.Type.DOUBLE:

self._goal_t = param.value

else:

self.get_logger().warn(f'{self.get_name()} Invalid parameter {param.name}')

return SetParametersResult(successful=False)

self.get_logger().warn(f"Changing goal {self._goal_x} {self._goal_y} {self._goal_t}")

return SetParametersResult(successful=True)

def main(args=None):

rclpy.init(args=args)

node = MoveToGoal()

try:

rclpy.spin(node)

except KeyboardInterrupt:

pass

rclpy.shutdown()

if __name__ == '__main__':

main()

## Question 4